Backpack Drone Detector& Jamming Integration

Model No.: TTSKBPDJ

- Overview

TTSKBPDJ is a portable jamming detection system with comprehensive drone monitoring capabilities. It monitors the model and operating frequency of drones within the range through data mining and in-depth analysis of drone signals, and performs signal jamming on its own.

- Product pictures and composition

Table 1 Product Hardware Composition Details

| Serial Number | Device Name | Number | Unit |
| --- | --- | --- | --- |
| 1 | Device host | 1 | pc |
| 2 | RF antenna | 10 | pcs |
| 3 | GPS antenna | 1 | pc |
| 4 | power adapter | 1 | pc |

Table 2 Product performance indicators-direction finding mode

| NO. | Item | Technical parameter |
| --- | --- | --- |
| 1 | Work pattern | Passive detection |
| 2 | Work frequency | 400M, 900M,1.2G,2.4G, 5.2G,5.8G and other commonly used drone frequency |
| 3 | Detection and identify UAV type | DJI series, FPV, FEMI, YUNEEC, AUTEL, HUBSAN, Powervision, Tello UAV and video transmission module |
| 4 | Detection range | 100-2000m (urban environment, tested by DJI AIR2 target) |
| 5 | Probe azimuth | 0 °~360° (omnidirectional antenna) |
| 6 | Recognition time | ≤2s |
| 7 | False alarm rate | 1 time / day |
| 8 | Capacity of discernment | The number of drones that can be simultaneously detected and identified ≥ 10 (5 manufacturers) |

| No. | ITEM | Technical parameter |
| --- | --- | --- |
| 1 | Work pattern | Directional passive detection |
| 2 | The object of action | DJI (Phanton and Mavic), AUTEL, and HUBSAN |
| 3 | Work frequency | 2.4G,5.8G |
| 4 | Detection range | 100-2000 m (urban environment with DJI AIR2) |
| 5 | Probe azimuth | ≤30° (UAV does not change the frequency during the locked DF mode and is 1 km from the device) |

Table 3 Product performance Jamming mode

| Channel | Working frequency |  | Output power(±1dBm) |
| --- | --- | --- | --- |
| CH1 | 420-450MHz |  | 47 dBm(50W) |
| CH2 | 700-840MHz |  | 47 dBm(50W) |
| CH3 | 840-930MHz |  | 47 dBm(50W) |
| CH4 | 1550-1610MHz |  | 47 dBm(50W) |
| CH5 | 1100-1300MHz |  | 47 dBm(50W) |
| CH6 | 5150-5300MHz |  | 45 dBm(30W) |
| CH7 | 2400-2500MHz |  | 47 dBm(50W) |
| CH8 | 5700-5900MHz |  | 47 dBm(50W) |
| Total output power: 380W |  |  |  |
| Power consumption : 900W |  |  |  |
| Power supply: AC90-260V, DC24-28V |  |  |  |
| Main engine weight : 12Kg (Manpack 10KG,Antenna 1.5KG,Control Box 0.3KG) |  |  |  |
| Size:520*400*230mm |  |  |  |
| With omni antenna 8 pcs |  |  |  |
| Jammer Module Number :8 |  |  |  |
| Cooling System: |  | Systematic Smart Cooling System |  |
| Jamming Source : |  | Noise Technology |  |
| Humidity: 30%-95% |  |  |  |
| Running temperature : -20℃ to +60℃ |  |  |  |
| Inside battery working 1.5h: 24V,25AH |  |  |  |
| Battery charger: 5A(Open the external switch to charge, automatically manage the charging current, full automatically disconnect) |  |  |  |
| Jamming Range: 800-1500m |  |  |  |
| External Remote control Panel : shows the battery voltage, can switch on/off each band. |  |  |  |